Customer : {{ client_name }}

Quotation No : {{ quote_reference }}

Site: {{ site_address }}

Item No : {{ item_no }}

Date : {{ date }}

Work Area : {{ work_area }}

DATE OF ERECTION: {{ date_of_erection }}

| Hazard | Persons Affected | Risk Assessment | Preventive/Control Measures | Residual Risk Assessment |
| --- | --- | --- | --- | --- |
| Contact with delivery or collection vehicles. | Scaffolders, supervision, site personnel and the general public | Moderate | ● Materials to be delivered/collected from site by HGV utilising an agreed traffic route. Reversing vehicles will be controlled. | Low |
| Fall of person during dismount from the delivery vehicle and during unloading/loading operations | Scaffolders, supervision and HGV driver | High | ● Operatives to observe ground conditions and surroundings before dismounting in a controlled manner, under no circumstances is the operative to jump from the vehicle. ● Demount demountable load beds, access raised load bed by ladder, use common site loading/unloading provision. ● Scaffolders will assist the HGV driver, when common provision is not provided by customer, Scaffolders follow SG4:22 to install single guard rail protection to load bed, use fall arrest equipment provided. | Low |
| Fall of materials during unloading, loading, movement on site and during erection and dismantle. | Scaffolders, supervision, site personnel | Moderate | ● The work area is to be physically barriered off and identified by signage or look outs posted to prevent 3rd party access. ● Only personnel that are directly involved in the operations should be in the work area. ● Plant to be operated by trained personnel. ● Unloading and loading to be supervised to ensure material is not accidentally dislodged. ● Materials to be stored by suitable means to prevent inadvertent movement i.e. scaffold fittings to be bagged. ● Materials must not be stored vertically unless they are prevented from falling. ● Suitable protection to be provided when working adjacent to public and site access routes. ● Personnel to wear safety helmets and safety boots. | Low |
| Manual handling during unloading/loading and during movement of materials on site. | Scaffolders | Moderate | ● Materials to be unloaded/loaded into an agreed storage area by crane or man handled. ● Materials to be raised or lowered by passing hand to hand or utilising a gin wheel and or rope. ● When handling materials good manual handling technique to be utilised. | Low |
| Trips, slips and falls whilst moving around site. | Scaffolders and supervision | High | ● Operatives to be aware of their surroundings, ensure they are looking where they are walking and working, do not step onto or over debris, avoid tripping or slipping hazards. ● Choose an access route or work area that is free from hazard. | Low |

_____________________________________________________________________________________________________________________________________

|  |  |  | ● Be aware of plant movements and maintain a safe distance. Report any hazard to your Supervisor. |  |
| --- | --- | --- | --- | --- |
| Abrasion, splinters from handling scaffold boards, lubricant from handling scaffold fittings | Scaffolders | Moderate | ● All site personnel to wear gloves whilst handling scaffold materials. | Low |
| Trips and slips over our materials whilst being stored on site or whilst being used during erection/dismantle. | Scaffolders, supervision, site personnel | Moderate | ● Material storage and work areas will be physically barriered off and identified by signage to prevent access, materials within such areas will be subject to good housekeeping. ● The provision of suitable storage areas is subject to agreement with site management. | Low |
| Fall of persons during erection and dismantle. | Scaffolders and supervision | Moderate | ● Scaffolders to wear a full body safety harness equipped with a 1.75m shock absorbing lanyard. ● The harness to be used in accordance with current NASC guidance note SG4:22. ● The Safety Step will be the preferred means of achieving collective protection. ● Rescue will be in accordance with the hierarchy of recovery actions detailed in the fall recovery plan. In most cases recovery can be undertaken from the existing scaffold, with the lift adjacent to the fallen person being boarded out and the person then pulled onto the lift. Where the lift is not guard railed, the rescuers to be clipped onto the scaffold structure via their safety harness. | Low |
| Fall of materials during cutting operations | Scaffolders, supervision, site personnel and the general public | Moderate | ● Operatives must strive to use the correct sized materials for the job i.e. if a 6 ft tube is needed don't use an 8 ft tube etc. ● Where scaffold materials in a scaffold structure need to be cut, the following to apply: o the material to be cut and discarded must be tethered. or o the cutting is a 2 man job, one cutting and one holding the off cut. | Low |
| Collapse of scaffold during erection and dismantle | Scaffolders, supervision, site personnel and the general public | Moderate | ● Scaffolds to be erected and dismantled by suitably qualified operatives who hold relevant CISRS registration cards. ● Scaffolds will be erected in accordance with The Work at Height Regulations 2005 and either {{ compliance_standard }} compliance sheet or a bespoke scaffold design. ● The scaffold and ties will be erected/dismantled progressively. ● Material storage on the scaffold will not exceed the loading capacity of the scaffold. | Low |
| Access to scaffold during erection prior to formal | Site personnel and the general public | High | ● On commencement of erection and dismantle an “incomplete” scaffold sign will be displayed adjacent to the access routes. | Low |

_____________________________________________________________________________________________________________________________________

| inspection handover and during dismantle. |  |  | ● Physical barriers to be erected (by others) to prevent access. |  |
| --- | --- | --- | --- | --- |
| Collapse of scaffold when erected. | Site personnel | Moderate | ● Customer to use the scaffold in accordance with the loading specified on the hand-over certificate/scaffold tag. ● No modifications are to be made to the scaffold, especially removal of ties or structural members. | Low |
| Contact with scaffolding when erected. | Scaffolders, supervision, site personnel | High | ● Tube and fitting protection caps to be installed where required. ● Hoarding and fenders to be installed (by others). | Insignificant |
| Environmental conditions (wind, rain etc) | Scaffolders and supervision | Moderate | Supervision will monitor the environmental working conditions to ensure that personnel are not put at risk. Wet weather gear may be utilized, however supervision will determine when work will cease due to environmental conditions. Noise, may be an issue when work is outside of the agreed site working hours. | Low |
| Substances hazardous to health | Scaffolders, supervision and site personnel | Moderate | ● {{ firm_name }} do not plan to use any COSHH substances on site. ● Customer to advise if there are any COSHH substances within our work area and provide relevant COSHH assessments. ● Personnel to comply with the relevant control measures. | Low |
| Environmental impact from our scaffolding operations | Scaffolders, supervision, site personnel and the general public | Low | ● Scaffold materials will be removed from site on completion of our works. ● Packaging will be segregated into the appropriate onsite disposal bins or be removed from site. | Insignificant |
| Permit for work hazards such as confined spaces, hot work, roof work, electricity etc. | Scaffolders and supervision | High | ● Supervision to establish whether work is subject to the customers permit for work system. ● Supervision and operatives to follow the control measures specified in the permit for work. | Low |

| Risk Assessment By : | {{ prepared_by_name }} | Signature : {{ prepared_by_signature }} |
| --- | --- | --- |

Notes :

i. When completing the risk assessment, please be aware what the definitions of Hazard and Risk are - Hazard “Something with a potential to cause harm” Risk “The likelihood that harm from the hazard will take place”

ii. For the Risk Assessment and Residual Risk Assessment, please use the terms High, Moderate, Low and Insignificant to reflect the level of risk.

iii. Where the Residual Risk Assessment is High or Moderate, the control measures identified are inadequate and you will need to consider alternatives to reduce the risk assessment to Low or Insignificant.

_____________________________________________________________________________________________________________________________________